Homework2——8位双向移位寄存器

- VHDL语言

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity jicunqi is

Port (

y:out std_logic_vector(7 downto 0);

x:in std_logic_vector(7 downto 0);

lod:in std_logic;--load

clk:in std_logic;--clock

l:in std_logic;--left

r:in std_logic;--right

reset:in std_logic;--clear

en:in std_logic;--使能端 低电平有效

cs:in std_logic);--片选 低电平有效

end jicunqi;

architecture one of jicunqi is

signal q:std_logic_vector(7 downto 0);

begin

process(reset,lod,clk,l,r)

begin

if cs='1' and en='1' then y<=q;

else

if reset='0' then q<="00000000";

elsif clk'event and clk='1' then

if (lod='1') then q<=x;

elsif (r='1') then

for i in 7 downto 1 loop q(i-1)<=q(i);

end loop;

q(7)<=r;

else

for i in 0 to 6 loop q(i+1)<=q(i);

end loop;

q(0)<=l;

end if;

end if;

y<=q;

end if;

end process;

end one;

- 激励文件

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity jicunqitb is

end jicunqitb;

architecture Behavioral of jicunqitb is

component jicunqi port(

y:out std_logic_vector(7 downto 0);

x:in std_logic_vector(7 downto 0);

lod:in std_logic;

clk:in std_logic;

l:in std_logic;

r:in std_logic;

reset:in std_logic;

en:in std_logic;

cs:in std_logic);

end component;

signal x:std_logic_vector(7 downto 0);

signal y:std_logic_vector(7 downto 0);

signal lod:std_logic;

signal clk:std_logic;

signal r:std_logic;

signal l:std_logic;

signal reset:std_logic;

signal en:std_logic;

signal cs:std_logic;

begin

dut:jicunqi port map(

lod=>lod,

clk=>clk,l=>l,r=>r,

reset=>reset,x=>x,

y=>y,

en=>en,cs=>cs);

process

begin

en<='0';

cs<='0';

clk<='0';

wait for 10 ns;

clk<='1';

reset<='1';

wait for 10 ns;

--load

clk<='0';

wait for 10 ns;

clk<='1';

x(7 downto 0)<="11010001";

lod<='1';

wait for 10 ns;

lod<='0';

--left

clk<='0';

wait for 10 ns;

clk<='1';

l<='1';

wait for 10 ns;

l<='0';

--right

clk<='0';

wait for 10 ns;

clk<='1';

r<='1';

wait for 10 ns;

r<='0';

--clr

clk<='0';

wait for 10 ns;

clk<='1';

reset<='0';

wait for 10 ns;

reset<='1';

end process;

end Behavioral;

- 仿真波形